Cochlear Implant Nerve Model Quick Reference

# What is CINM?

The cochlear implant nerve model (CINM) is a population model of the auditory nerve spiking in response to electrical stimulation. It was developed by Hamacher (2004) and re-implemented in MATLAB by Fredelake and Hohmann (2012).

## What can I do with it?

Currently the model can be used to compute a monaural internal representation of an auditory stimulus which was processed by a cochlea implant. You can also predict monaural speech intelligibility and model measured ECAP-Amplitudes.

## What this documentation is and what not

This documentation is intended to give you a short overview on how to use the model, e.g., how to install it and which Matlab-script to call first. If you want to know how the model works in all its  details have a look at the “Architecture CI Nerve Model.docx”-file.

# Installation

Simply download the latest version from Github: https://github.com/tjurgens/CImodel/archive/master.zip

Extract the archive and place it anywhere on your Computer. To start using/working with the model, run TestCIModel.m located in the subfolder GUI.

If you want to perform speech intelligibility prediction you must have a folder named “CImodelresults” in the same directory as the “CI nerve model” folder. The “CImodelresults”-Folder must contain the single words of the german sentence test “oldenburger satztest” (OLSA).

# Demos

Run TestCIModel.m in subfolder GUI to explore the world of CINM.

There are several demos of the model in this GUI:

- To simulate the signal processing of a Cochlea Implant and the post-processing of the auditory nerve, click the “run”-Button. This will perform the Simulation and display several figures like the stimulation pattern across the CI-Electrodes or the spiking potentials in the auditory nerve

- To see some characteristics of the model, like forwards masking in the auditory nerve click the buttons in the reproduce-Panel

- If you want to use the model to simulate ECAP-Measurements, feel free to hit the buttons in the ECAP-Panel.

- If speech intellibillity predicition is more fun to you, try the Buttons in the Speech Intelligibillity predition-Panel. Please note, that you must press them in the right order (from one to three) to get a result in the command window.

# User Code

The following sections will give you some hints how to change the model to your own needs.

### How to process custom wav-Files with the model

Simply put all custom wav-Files in the subfolder wavfilestore. After that use the drop-down-menu in the Test_CIModel.m-GUI to choose the correct wav-File. Currently only the wav-audiofileformat is supported. If you have a function, which can read other audiofileformats, replace the wavread-call in main_CI_SP_AN.m with your own function call.

### How to customize the basic parameters of the auditory nerve model

All basic parameters are stored in the struct ANParams, which is defined in the file set_global_parameters.m. Save this file under a different filename and place it in the subfolder parameterstore. After you adjusted the parameters, you can run the model with your customized parameters by choosing the correct parameterfile in the drop-down-menu. Currently changing the basic parameters will only have effect on the model processed by clicking the “Run”-Button.

### How to customize the basic parameters of the Cochlear Implant processing

All Cochlear Implant processing parameters are stored inside the struct CIParams. You can alter some parameters of the Cochlea Implant processing by defining the parameters in the m-File you want to run before the call to ACE_signal_processing.m. You can find all adjustable Parameters in the file set_global_constants_ACE_demo.m in the subfolder ACE.

Please note that you must use exactly the same variable names, in order to overwrite the default Parameter values.

Example for setting a different CI processing parameter:

% create signal

load(‘Handel’);

%set custom CI Parameter, e.g. a different pulse rate:

CIParams.pps = 1200;

[mINew,pps,maxima] = ACE_signal_processing(y,Fs,CIParams);

Introducing more parameters to the Model

This is simple, if you want to add a Parameter, just add them to the structs CIParams (defined in set_global_constants_ACE_demo.m, subfolder ACE) or ANParams (defined in set_global_constants.m, subfolder parameterstore). Now you can access them in every function, which also has access to these structs.

### Introducing another stage in the model:

Please read the Technical Documentation to get to know which stages are already there. Then have a look at the code and try to figure out where to put your own function.

The following rough overview of the most important files will help you to get startet:

Main_SP_CI_AN_model: The easiest entrypoint: You can see which functions are used to simulate the Cochlea Implant signal processing, how the auditory nerve model is called and how an internal representation is plotted.

Main_CI_AN_model: This function simulates the auditory nerve processing.

| Model stage | M-File name |
| --- | --- |
| Cochlear Implant signal processing | ACE_signal_processing.m in subfolder ACE |
| Spatial Spread Function | calculate_AVF.m in subfolder Parameter_helperfunctions |
| Cell membrane | generateChronaxieRheobase.m in subfolder Parameter_helperfunctions |
| Membrane noise | membraneNoise.m, auditoryNerve.m |
| Refractory behavior | calculate_refracConstants.m, calculateRefractoryFunction.m |
| Latency and Jitter | calculate_latency_dj.m, auditoryNerve.m |
| Central auditory processing | main_CI_AN_model.m |
| Internal representation | main_SP_CI_AN_model.m |

### Running custom experiments with the model (without GUI)

If you want to investigate the influence of your own parameter or have added a custom stage to the model, you want to run the model several times with different inputs. Currently the GUI does not support this feature in a convenient way, you must write your own Matlab-script. An example script can be found in the examples folder. Please note, that you must add the CI-model folder to you Matlab-PATH in order to run the example script.